	[image: image1.png]aifn




	BHARAT HEAVY ELECTRICALS LIMITED

Heavy Power Equipment Plant

Hyderabad – 502032. INDIA

Phones: 23182250/23020173
Fax No.s : 040-23021910 & 23021954,23186173
E-Mail : burre@bhelhyd.co.in
	Date:

Due Date:

Supplier Qtn. No. :

Date :
	___________________

___________________

___________________

___________________

	TENDER DOCUMENTS TO BE FILLED BY THE SUPPLIER FOR SUPPLY OF 

12 TON BOTTOM POURING LADLE

	ADDRESS OF THE SUPPLIER:



	TELEPHONE NO.S :



	FAX NO.S :



	E MAIL ADDRESS :

	Sl. No.
	DESCRIPTION OF BHEL REQURIMENT
	BHEL SPEC.
	OFFERED
	DEVIATIONS
	REMARKS

	1.0
	PURPOSE

Bottom pouring ladle of 12 Ton capacity is required to handle molten  Steel and to carry out pouring operations in Foundry.
	
	
	
	

	
	
	
	
	
	

	2.0
	TECHINICAL DETAILS
	
	
	
	

	2.1
	Capacity of Ladle
	12 Ton
	
	
	

	2.2
	Quantity required
	1 No.
	
	
	

	2.3
	Type of Ladle
	Bottom pouring ladle with gear box assembly
	
	
	

	2.4
	DIMENSIONS

Supplier has to get the dimensions of the ladle wetted by M/s BHEL after placement of Order and adopt the necessary changes if any.

The tentative dimensional details of the ladle can be as follows
	
	
	
	

	2.4.1
	Shell Dimensions
	
	
	
	

	
	Inside Dia
	1830 mm
	
	
	

	
	Bottom Dia
	1600 mm
	
	
	

	
	Height
	1830 mm
	
	
	

	2.4.2
	Plate Thickness
	
	
	
	

	
	Side
	16 mm
	
	
	

	
	Bottom
	20 mm
	
	
	

	2.4.3
	Refractory Lining
	
	
	
	

	
	Side
	165 mm
	
	
	

	
	Bottom
	215 mm
	
	
	

	2.4.4
	Top Clearance
	390 mm
	
	
	

	2.5
	Details of Graphite Stopper Head

Supplier has to Specify
	
	
	
	


	3.0
	FEATURES :
	
	
	
	

	3.1
	Bottom pouring ladle shall be supplied with Stopper Rods and Nuts (2 No.s. ), operating lever arrangement, gear mechanism with hand wheel for tilting the ladle, trunions, bail-arm, heat shield, hook / shackle for lifting with EOT crane lop, legs, refractory lining ring, locking lever etc.
	
	
	
	

	3.2
	Bottom pouring ladle shall be supplied with 5 No.s of graphite stopper heads as optional item.
	
	
	
	

	
	
	
	
	
	

	4.0
	DOCUMENTATION:
	
	
	
	

	4.1
	Supplier shall furnish constructional details of the Ladle. i.e. General arrangement drawing, volume to weight calculations for molten steel along with the quotation.
	
	
	
	

	4.2
	Supplier shall furnish detailed dimensional sketches of stopper rod with nut, refractory insulating sleeve to be fitted on to the stopper rod, refractory nozzle and graphite stopper head.
	
	
	
	

	4.3
	Three sets of each of the following documents to be furnished along with the Ladle
	
	
	
	

	
	Operation Manual
	3 sets
	
	
	

	
	Maintenance Manual
	3 sets
	
	
	

	4.4
	List of customers for similar geared bottom pouring ladle shall be furnished along with the offer.
	
	
	
	

	
	
	
	
	
	

	5.0
	PRE INSPECTION :

M/s BHEL Engineers will inspect the Ladle at suppliers works before dispatch.
	
	
	
	

	
	
	
	
	
	

	6.0
	GUARANTEE :

The geared bottom pouring Ladle shall be guaranteed for a period of one year from date of commissioning.
	
	
	
	

	
	
	
	
	
	

	7.0
	SPECIAL REMARKS:
	
	
	
	

	7.1
	Vendor shall submit technical offer and price offer separately in two sealed covers
	
	
	
	

	7.2
	They shall be original equipment manufacturers
	
	
	
	

	7.3
	They shall be in line of manufacturing of similar Bottom pouring ladles  with capacities greater than or equal to 5 Ton and up to 20 Ton for not less than 5 years.
	
	
	
	


