BHARAT HEAVY ELECTRICALS LIMITED

UNIT: JHANSI    

(WORK. ENGG. & SERVICES DEPTT.)

EQUIPMENT


:
PORTABLE DRILLING MACHINE

ITEM NO



:
MT-3/3/1447

LOCATION



:
LOCOMOTIVE

QUANTITY



:
1 NO.

SPECIFICATION


:
6022/Rev-2
PORTABLE DRILLING MACHINE

Portable universal radial drilling and tapping machine mounted on bogie bed should be capable of drilling & reaming of holes up to 50 mm dia (max) in steel. The column to be mounted on bed which slides on the bogie bed and can be traversed by hand wheel & powered motor. It should have provision of locking rigidly in any position on bed. The column should be of robust design so that no tendency of vibration occurs during high speeds and loads. The radial arm should be suitably designed so that it can be traversed across the column by means of a hand wheel & powered motor, elevated by hand wheel & powered motor and moved radially through a complete circle. The angle of drill head swivel from horizontal upwards & downwards should be 90 degree. Secure locking provision should be provided for all the movements. Drilling head to be adjustable in vertical and horizontal planes to any angle and locked securely in any position. The  arm & drill head rotation should be 360 degree. Spindle of the drilling head should be of good wear resistant and case hardened and ground. Power traverse, fine, coarse, hand adjustment and reversing motion should be provided to spindle.      

All gears of the spindle drive should be case hardened and ground on the tooth flanks. All shafts in the drive system should  run on roller bearings.

All scales should  be calibrated in metric units.

SPECIFICATION FOR  PORTABLE DRILLING  MACHINE WITH STANDARD ACCESSORIES.

(1) Swing Radius of drilling spindle (max) 


=   2270 mm

(2) Swing radius of drilling spindle (min)


=   1420 mm

(3) Distance of Spindle end from floor 

      when horizontal drilling(max)



=   2725 mm

(4) Distance of Spindle end from floor

      when horizontal drilling(min)



=   295 mm

(5) Spindle feed traverse




=   350 mm.

(6) Spindle Speed






=   15(16 to 800 rpm)

(7) Feed


=   0.05 to 0.5 mm in six                                                                                          

                                                                                                     graduated segments

(8) Spindle Bore






=   MT-5 (Morse Taper) 

(9) Traverse of column in bogie



=   1200 mm

(10) Size of bed






=   1050 x 2610mmX260MM

(11) Drilling & reaming capacity                                           =  Upto 50mm.dia (max)

(12) Machine should be suitable for Power supply-415 Volts, 50 Hz,3-phase AC

(13) Paint of M/c                                                                     =  Verdigris green (IS-5/1961)

Note:-  All standard accessories should be supplied with the machine as a part of machine at no extra cost

Following to be submitted with the offer:

1. Dimensioned outline general arrangement drawing.

2. Details of safety devices / guards provided.

3. Electrical schematic diagram.

4. Total electrical load.

5.   Quotation with item wise prices of spares for 2 years normal working.

6.   Details of standard & optional accessories offered.

      7.   Catalogues / leaflets of machine offered   

       8. Quotation for Erection & commissioning of the offered machine.

QUALIFYING CONDITIONS


Only those vender, who have supplied and commissioned portable machine tools with drilling facility in the past ten years and such machine is presently working satisfactorily for more than one year (more than six month if supplied to BHEL) after commissioning should quote. The following information is to be submitted by the vender about the companies where similar machines have been supplied. This is required from all venders for qualifications of their offers.

1. Name of the customer/ company where similar machine/ crane is installed.

2. Complete postal address of the customer.

3. Year of commissioning.

4. Name and designation of the contact person of the customer.

5. Phone, Fax No. and e-mail address of the contact person of the customer.

6. Performance certificate from the customer regarding satisfactory performance of      

      the machine/ crane supplied to them.

	Date
	Prepared By
	Checked By
	Approved By

	
	Dy. Mgr (WEX)
	DGM(WEX)
	AGM.(WEX)
	Sr. Mgr (LMM)


