BHARAT HEAVY ELECTRICALS LIMITED

RAMACHANDRAPURAM::HYDERABAD – 502 032::502 032 INDIA

Phone No: 0091- 040- 23182250, 23020173

Fax No: 0091- 40- 23021910, 23020173

SPECIFICATION & SCOPE FOR PORTABLE

WELDING FUME & DUST EXTRACTOR
Qty: 15 Mtrs. Hose 1 No.
1.0 Application:-
-
To extract welding fumes at source of welding in constricted place, 
             Absorb all harmful elements in filtration system & release clear air.

· Welding fumes are required to be captured from various welding stations in different work centers. 
2.0 Technical Specifications :-

 i)
Net Extraction Capacity 
-
1250 m3 /hour

ii)
Motor Power


-
0.75 KW / (max)


iii)
Extraction Hose Length
-
15.0 mtrs. 
iv) Cartridge type filtration system with proper surface treatment , having    

            Filter surface efficiency
-
99.8%
v)
Filter surface area

-
> 30 Sq. mtrs.
vi)        Free blowing capacity of fan
-
> 2200m3/hr.

vii)
Automatic filter cleaning with compressed air pulse & with set frequency of pulses.
viii)
Automotive Start /stop switching system.
ix)
Built in compressor for independent filter cleaning.
x)
Aluminum mesh (high grade) pre-filter to protect the cartridge from sparks.

xi)
Noise Level


-
within 69 dB (A).

xii)
Audio alarm for filter saturation and over heating.

xiii)
Input Supply 


-
220 V ± 10% Variation single phase 
& 50 HZ ± 3% Variation


xiv)
Working Supply 

-
Ambient Temperature - 3°C to 48 °C








Relative humidity
- Upto 95 % 

                                                                                         During rainy Season.                                                                                                              


xv)
Filter Type


-
Self Cleaning 
3.0
Technical Feature /Condition :-
 i)
Steel, Welded Construction painted with polyester –epoxy, Powder paint.
ii) Door for easy access to the filter and for waste drawer. It should be tightly sealed with latches.

iii) Pre-filter, Metal filter to protect cartridge filter from spark & for first step of filtration.
iv) Cartridge –main filter on roll-out tray.

v) Carbon filter.
vi) Handle 

vii) Swivel Castor Wheel for mobility.

viii) Waste drawer mounted on roll-out rails.

ix) Built in Noise Abatement features.

x) Built in safety feature for safe use as per National / International norms.

xi) Rigid construction for day to day shop floor use.

xii) Lifting eye for handling by crane.

xiii) Dia of flexible hose
: 
> 200mm

xiv) Flexible Hose should have manufactured with synthetic material and external metal spiral reinforcement with wear resistant material. 

4.0      Scope of supply:-
i) Portable fume extractor as per specification and technical features.

ii) 15.0 meters long heat resistance flexible metallic braided hose with magnetic foot.

iii) To be mounted on castor wheel base for portability.

iv) Input supply lead- 10.0 m long with plug.

v) All connectors, clips etc.  required for hose and connection.

vi) Recommended consumables and spare for 2 year smooth operation.
5.0         Commissioning and trials by vendor at BHEL works.

6.0         Training to BHEL personnel
7.0         Guarantee for 3 years smooth and trouble free working after successful 
              commissioning.
8.0       Prompt after sales service and response time within 24 hours.

9.0       3 sets of operation and maintenance manuals consisting of following documents 
             With supply:-

i) Functional details with construction details.
ii) Specification and makes of main components.
iii) Operating instructions.

iv) Maintenance instructions.

v) Trouble shooting and remedies.

10.0 All clarification to be given point wise as per above requirements and supporting    

technical literatures to be submitted with offer.

11.0 Provide names & addresses of satisfied customers using similar Welding Fume                                             

            Extraction Systems.

********

