BHARAT HEAVY ELECTRICALS LIMITED

RAMACHNADRAPURAM:: HYDERABAD- 502 032

TENDER NOTICE

Ref No: HY: M&S: P&C: Contracts:2006:735 
       Dt: 24th    February  2006

Name of the Department:  
 M&S (Planning & Co-Ordination)

TENDER NOTICE NO: HY/M&S/P&C/2005-06/059

To

Sealed tenders in two bids (Techno commercial & Price bids) in a sealed cover, subscribing the tender notice number, tender date, name of the work and due date of tender opening will be received by the undersigned from the contractors satisfying the prequalification requirements indicated below and will be opened at 14.30 hrs on the due date itself in the presence of tenderer’s or their authorized representatives in purchase co-ordination  Department, BHEL, Ramachandrapuram, Hyderabad.

	Name of the Work
	Earnest Money Deposit ( EMD)
	LAST DATE

FOR DOWN LOAD
	Due date / Time for receipt of tender
	Date and Time of Tender Opening
	Period of completion

	AMC & Service contract for Fire Alarm, Detection and Halon Gas Extinguishing System at GT Testing, computer building, telephone exchange, 06 Annexe, MRS & 10 panel room in BHEL Factory..

 Schedule of work  - Annexure-2 is enclosed
	Rupees

10,000/-    
	20.03.2006
	21.03.2006 at 13.30 Hrs
	21.03.2006 14.30 hrs
	TWELVE MONTHS


Estimated amount: Rs3.47 Lakhs

Prequalification requirements:

1. PAN No.  

2. Experience in maintenance of fire alarm systems. 

3. EMD payment of Rs10,000/-

4. ESI Code, Labour Licence, PF code 

SUBMISSION OF BIDS (PROCEDURE):

The bids shall be submitted in two parts 

Part A: Techno-Commercial Bid  
Part B:   Price Bid.

Details in  part   A:    The first envelope shall contain the techno-commercial bid with  following documents :

       1. Work experience documents, as mentioned in pre-qualification requirements at least 2 copies of executed work orders/ work experience in the last 4 years from any reputed firm, value of  each order should be approx  Rs 4 lakhs.

2. Statutory codes : copies  of ESI ,PF, valid Labour Licence, PAN  No.

3. Licence Copy from fire safety services dept ( optional).
4. Receipt of EMD paid in BHEL cash office/ DD/ BC

5. Acceptance of  terms and conditions as per the enclosed  pro-forma 

6. Techno-Commercial Bid
7.  Applicable taxes if any should be furnished in the techno commercial bid.
This envelope shall be sealed and super scribed with “Techno-Commercial Bid for     “AMC for Fire Alarm System  - Annexure-I”  

Details in part B:     The second envelope shall contain only the Price bid in  Annexure – 2( 2/1 to 2/7,       No other information except price will be considered. 
The envelope shall be super scribed with “ Price Bid for Fire Alarm System in BHEL”

 Both the above two envelopes shall be kept in another sealed cover and should     be dropped in tender box placed in 02 Annexe/ GF. 
 From address should  appear on the cover:   
 Sealed cover must be addressed to : Manager/ M&S (P&C), 02 Annexe,

             M/s BHEL, R.C.PURAM, HYDERABAD- 502 032

Tender No should be written as : HY/M&S/P&C/2005-06/059 &   Tender due  date as : 21.03.2006
Note:   Contractor should see that all pages are signed and rubber stamped before dropping in tender box.

Tender should  reach well in advance but not later 13.30hrs of 21.03.2006. BHEL will not be responsible for any postal delays. The tenders will be opened on 21.03.2006 at 14.30 hrs. In unforeseen conditions tender can be extended and next due date will be intimated through post.   Price bid opening date will be intimated in advance to only TECHNICAL   qualified tenderers.  Tenderer should fill rate for all items otherwise tender will get rejected. 
Thanking you,






Yours sincerely,

For and on behalf of

Bharat Heavy Electricals Limited

(G.Janardhan Rao)

Manager/ M&S (P&C)

02 Annexe, Ground Floor

PHONE: 040-2318 2739

FAX: 040- 2302 0185

Email ID: gjrao@bhelhyd.co.in


ANNEXURE-I

BHARAT HEAVY ELECTRICAS LIMITED

RAMACHANDRAPURAM:: HYDERABAD-502 032

M&S DEPARTMENT / PLANNING & CO-ORDINATION

TENDER SCHEDULE FOR THE WORK OF   “AMC & Service Contract Fire Alarm, Detection and Halon Gas Extinguishing System at GT Testing, Computer Building, Telephone Exchange, 06 Annexe, MRS & 10 Panel Room in BHEL Factory
TECHNO- COMMERCIAL BID (PART-I)

THIS IS AN INDIVISIBLE WORKS CONTRACT AND THE CONTRACTOR SHALL QUOTE FOR ALL ITEMS INDIVIDUAL RATES

 Work: AMC for fire alarm system

This tender schedule must be submitted along with the General Conditions, Special conditions etc. enclosed herewith duly signed and stamped on each page with your covering letter.  Remarks  should be TYPED/ hand written   on tender schedule  only.  Any deviations / deletions/ additions  etc.  can be neatly typed  on your letter head and can be  enclosed  with  techno- commercial bid  document.

To be filled up by the Bidder

Description




          Acceptance/ Remarks by the  Contractor .

	1.
	PF CODE NO:
	

	2.
	ESI CODE NO:
	

	3
	LABOUR LICENCE NO & 

Validity:
	

	4
	PAN No:
	

	5
	 AMC period   1 year
	

	6
	Offer Validity- minimum 90 days from technical bid opening date.
	

	7
	EMD  Rs10,000/-    Cash receipt/ cheque  Details
CLAUSE 21
	

	8
	Security Deposit clause:     Acceptance 

( Details see under terms and conditions clause: 22 )
	

	9
	Service Tax Regn  No:  ( if applicable) : 

Whether quoted rates are  inclusive or exclusive should indicate

 any kind of other taxes -  please specify for calculation/ loading purpose.
	

	  10
	PAYMENT TERMS:

Progressive payment on prorate basis for the   work completed in all respects and on submission of bill.  Bill should be recorded in measurement book with  full details of work carried out. Release of payment may take place about 30 days from the date of recorded in M-Book
	

	  11
	 No Advance payment will be released :
	

	  12
	PENALTY:  0.5% of the contract value per week or part thereof upto a maximum of 10% of the contract value shall be levied beyond the scheduled completion time.  Otherwise for not attending the work  in between ( monthly checkup) contract period  also will attract penalty.
	

	   13.
	Acceptance of  TERMS AND   Conditions; obligations of the contractor, schedule of work.
	

	   14.
	Decision of BHEL Representative shall be final in the matter of inspection at any stage
	

	15
	All items will be carried out as per Annexure-II 

2/1 to  2/7
	


Terms and Conditions ( TECHNICAL) :

1. The contractor shall depute their engineer once in every month and work as per the instructions of the site in charge. Also  for (i) Routine inspection, (ii) operational testing,    (iii) functional testing.

2. The contractor shall attend the complaints if any on top priority basis after registering the   complaint.    No extra charges will be paid for additional visits and contractor shall maintain and submit the log sheets for the approval of the engineer as and when they visit the site.

3. The maintenance contract should be strictly as per the  IS 2189 of 1988 and all other National & International Standards which are applicable for Fire alarm system in all respects.

4. The contractor shall  replace the defective items with only genuine original spares  after the approval of the site in charge.

5. The contractor has to test the system in total and shall issue a statement on every visit.

6. The contractor shall check entire wiring / cabling  for the system and shall replace if found  defective. Cable  for replacements  will be supplied by BHEL on free of cost.

7. The Contractor has to meet the indenting departments and obtain their signature on service report/ register  for periodical checking of fire alarm system
8. Mock tests should be conducted at half yearly intervals in the presence of Fire services department by the contractor and should obtain report of satisfactory performance from Fire Wing/ CISF.

9. The contractor shall employ qualified  and experienced technicians  and engineers in similar field for attending the AMC  work.   If any damage due to bad practice,  will be charged to contractor for the equipment.

10. Offer shall be inclusive of all necessary essential spares like ICS, Transistors etc.,

11. The offer is inclusive of un-limited breakdown calls with in 24 hours of reporting.

I. OBLIGATIONS OF THE CONTRACTORS   
1. If the tender is made by an individual, it shall be signed with his full name and his address shall be furnished.  If it is made by firm, it shall be signed with the co-partnership name by a member of the firm, who shall also sign his own name, also furnish the name and address of each member of the firm.  If the tender is made by a corporation, it shall be signed by a duly authorized Officer who shall produce with the tender, satisfactory evidence of his authorization.  Such tendering corporation/firm is required to furnish evidence of its existence along with bid.

2. The EMD will be refunded to the unsuccessful Tenderers on application after rejection of the tender is intimated.  The earnest money will be retained in the case of the successful tenderers and either of the cases will not carry any interest.  It will be dealt with as provided in the conditions attached to the tender.

3. Whenever a tender is to be accepted, the tenderer, whose tender is under consideration, shall attend the Office of " Officer Inviting the Tender " on the date fixed by written intimation to him.  He shall forthwith, upon intimation being given to him by the " Officer Inviting the Tender” for acceptance of his tender, complete the execution of the agreement by signing all documents connected therewith.  Failure to do so and not to commence the work within one month from the date of intimation shall entail forfeiture of the earnest money.

4. Tenderers shall persue carefully the instructions and directions to the parties given in the tender document and the conditions there of and all other relevant documents before quoting the rates for the work.  The approximate quantity of work to be executed under each class is given in the schedule.  The quantities are given with a view to enable tenderer to quote his overall rate for each class of work in the tender form and for a uniform comparison of tenders.  It shall be definitely understood that the schedule is liable to alterations at the discretion of accepting authority.

5. Tenderers shall keep the offer valid for a period of 90 days from the date of opening of tender.  After submitting the tender, he will not recall his offer or modify the terms and conditions thereof.  Should the tenderer fail to observe or comply with the foregoing stipulations, the EMD shall be forfeited.

6. Tenderers have to quote their rates in the tender schedule legibly written in figures and words and those not submitted in proper form are liable for rejection.

7. The submission of tender shall be strictly in accordance with the terms and conditions stipulated in this tender notice.  No counter conditions will be acceptable or valid.

8. Contractor shall be deemed to have included in his tender price of all the plant, machinery and appliances required for the purpose of all operations connected with the work embraced under the contract to secure a satisfactory quality of work and rate of progress which in the opinion of the " Contract Signing Officer" will ensure the completion of the work within the time specified.

9. BHEL is having every right to split the schedule and to award the work to single or more  parties on the lowest offered rates basis.

10. BHEL is having right to call the lowest bidder for price negotiations, if the rate or amount quoted by contractor is felt on higher side. In such cases contractor should attend negotiations with full details.  

11.  If BHEL feels to continue the contract, BHEL may request the contractor for extending the contract  for one more year with same terms and conditions.

12. The rates quoted by the contractor should be firm for the contract period.  There shall be no revision in contract rates during the period of the contract.  

13. BHEL reserves the right to terminate the contract at any stage without assigning any reason.

14. The contractor must have phone facility at his office/ residence or cellular phone for proper communication.

15. Withdrawal from contract during contract period in BHEL will entail forfeiture of security deposit.

16. The contract agreement shall be entered with BHEL, Ramachandrapuram,  Hyderabad on valid non judicial stamp paper of the value of Rs100/- to be purchased by the contractor at his own cost.

17. Labour engaged by the contractor should be disciplined & exhibit good behaviour in dealing with employees of BHEL. Any misbehavior or conduct of any Peron engaged by the contractor is not good, contractor shall change that person immediately or else it may even lead to termination of the contract & security deposit will be forfeited as penalty.

18. The Executing Officer shall ensure the execution of work on quantifiable basis taking into account equipment and machinery deployed rather than availability of technicians.

19. Disputes, grievances between the contractor and his labour, will have to be settled by the contractor only.

20. The labour employed by the contractor, if found in abetting any of fellow labour or contractors or any BHEL employee, the same shall be considered as an act of indiscipline.  Such laborers shall be removed from the services of the contractor, on the advice of BHEL.  Further the contractor shall initiate every necessary action in accordance with the relevant Laws, Rules & Regulations and enactments of state and Central Governments.

21. Earnest Money Deposit:  

Earnest Money is to be paid by each tenderer to ensure that the tenderer does not refuse to execute the work after it is awarded to him.  The rate of earnest money deposit shall be as per tender schedule. 

          EMD by the Tenderer will be forfeited as per Tender Documents if

         i)    After opening the tender, the tenderer revokes his tender within the validity period or increases his earlier quoted rates.

         ii)     The tenderer does not commence the work within the period as per LOI / Contract. In case the LOI / contract is silent in this regard then within 15 days after award of contract.

             EMD shall not carry any interest

22.    Security deposit:   

      Security Deposit should be collected from the successful tenderer.  The rate of Security Deposit will be as below:

Upto Rs. 10 lakhs



10%

The security Deposit should be collected before start of the work by the contractor.

Security Deposit may be furnished in any one of the following forms 

  i)       Cash (as permissible under the Income Tax Act)

 ii)        Pay Order, Demand Draft in favour of BHEL.

iii)
       Local cheques of scheduled banks, subject to realization.

i) Securities available from Post Offices such as National Savings Certificates, Kisan Vikas     Patras etc.        (Certificates should be held in the name of Contractor furnishing the security and duly pledged in favour of BHEL and discharged on the back).

ii) Bank Guarantee from Scheduled Banks / Public Financial Institutions as defined in the Companies Act subject to a maximum of 50% of the total security deposit value.  The balance 50% has to be remitted either by cash or in the other form of security. The Bank Guarantee format should have the approval of BHEL. 

iii) Fixed Deposit Receipt issued by Scheduled Banks / Public Financial Institutions as defined in the Companies Act. The FDR should be in the name of the contractor,  A/C BHEL, duly discharged on the back. 

iv) Security deposit can also be recovered at the rate of 10% from the running bills. However in such cases at least 50% of the Security Deposit should be collected before start of the work and the balance 50% may be recovered from the running bills.                                                                                                                                                                         

v) EMD of the successful tendered  shall be converted and adjusted against the security deposit.

             The security deposit shall not carry any interest

	    
	
	

	23.  WAGE SLIP, employment card    etc to the technicians/  supervisors/ engineers  have to
            be arranged by the contractor.
	

	24     PF Remittances should be made before 7th of succeeding month for workmen   engaged,  copy

	must be produced in HR (Personnel)  for obtaining HR Clearance  and also to the site incharge 

( as per EPF clause).

25. Contractor shall provide to his employees all tools, tackles and equipments to carry out the job under the 
contract at his own cost.

26. Contractor shall provide safety appliances at his own cost which may be required under
        the statute or otherwise.

27. Contractor shall provide all material at his cost as mentioned in the contract to his 
         employees for carrying out the job

28.   Payments will be made to the contractor on the basis of quarterly period,   carried out
         by him and on   submission of prorate  bill    basis.

29 Any future wage raise on account of DA Increase/ Wage revision by AP Government

 may be met by the contractor himself. BHEL will not reimburse the differential 

in rate of DA  or Wages. 

30. All payments will be subject to deduction of income tax at source as per 

Income Tax and Service Tax as per the applicable Rules 

( present rate of TDS  is 2.24%).


	
	
	


31.   The issue of materials is governed by the conditions specified in the schedule-II (If any).  All other materials shall be supplied by the contractor himself and the rates for the relevant items are inclusive of all such supplies.

32.      BHEL  reserve the right to deviate either by addition or by deletion from the schedule of items of work given in the tender document after awarding the work.

ANNEXURE-C

CONTRACTOR’S OBLIGATIONS

A) CONTRACTUAL

1. The Contractor will ensure that the employees deployed by him in the premises of BHEL are physically and mentally fit and do not have any criminal record. 

2) The Contractor will maintain records of his employees deployed to carry out the job.

3) The Contractor will provide employment card/Identity card with photograph duly attested by him to his employees.

4) The Contractor will provide uniforms / safety appliances to his employees.

5) In the event of termination of contract for any reason whatsoever, the contractor shall withdraw all his employees from the establishment of BHEL.

6) The Contractor will obtain insurance cover for his employees and take third party risk insurance coverage at his own cost.

7)
The Contractor will decide the number of employees to be deployed for execution of the work awarded to him and will be solely entitled to dictate such workers about the manner of carrying out the work. 

8)
The Contractor or his authorized representative will supervise the work allotted to him and being carried out by his employees or will post a Supervisor for this purpose.

 9)
The Contractor will be responsible for the good conduct of his employees.  In case of any misconduct / misbehaviour by any employee, contractor will replace such employee immediately.

10.
The contractor will ensure that the job is executed through the employees on his rolls and under no circumstances he will deploy any casual employee to carry out the job nor shall he sub-contract the job awarded to him.

11)
The Contractor will keep watch on his employees and he will be liable for any pilferage / loss to BHEL due to acts of omission and commission by his employees.  Similarly, liability for any compensation to outsiders on account of any act of omission and commission by the employees deployed by the contractor shall lie exclusively with him.

12)
The Contractor will ensure that all precautions are taken for the safety of his employees.

13)
The Contractor will provide to his employees all tools, tackles and equipments required to carry out the job under the contract at his own cost.

14)
The Contractor will provide safety appliances at his own cost which may be required under the statute or otherwise.

15)
The Contractor will provide all material at his own cost as mentioned in the contract to his employees for carrying out the job.

16.
spares  will be supplied by BHEL free of cost.  

B.
STATUTORY 
17)
All statutory requirements under Minimum Wages Act, 1948, Payment of Wages Act, 1936, Workmen Compensation Act. 1923, EPF & MP Act, 1952, Payment of Gratuity Act, 1972, ESI Act, 1948, The Contract Labour (R&A) Act, 1970, Payment of Bonus Act, 1965, Income Tax Act etc. shall be complied with by the contractor.

18)
The Contractor shall comply with all statutory requirements, rules, regulations, notifications in relation to employment of his employees issued from time to time by the concerned authorities.  

19)
The Contractor shall ensure payment of statutory prescribed minimum wages as applicable from time to time in the presence of authorized representative of BHEL and maintain proper records of their timely disbursement.  These records need to be preserved for a period of at least 3 years and made available even after the contract is over for any verification by  the statutory authorities.  

20)
The Contractor should have independent code numbers under EPF & MP Act, 1952 and ESI Act, 1948 and he shall cover his employees under the said codes.

21)
The Contractor shall provide PF pass book to his employees and ensure payment of PF, EDLI, pension dues under EPF & MP Act, 1952 to the RPFC.  

22)
The Contractor shall ensure payment of ESI contribution under ESI Act, 1948.  

23)
The Contractor shall produce proof of deductions as well as remittances of PF, EDLI, Pension, ESI contribution, administrative charges etc. wherever applicable and shall maintain proper records.

24)
The Contractor shall furnish proper returns to the concerned statutory authorities.  

25)
The Contractor shall be solely responsible for non payment/  delayed payment of wages, contributions under EPF & MP Act, ESI Act etc.  

26)
In case the contractor fails to make payment of wages to his employees or remittance of contribution to the concerned authorities, the security deposit/other dues under the contract can be utilized by BHEL to discharge the liability of the contractor.    

27)
The Contractor shall indemnify BHEL against all claims and losses under various statutes or any civil or criminal law in connection with employees deployed by him.

28)
The liability for any compensation on account of injury sustained by an employee of the contractor will be exclusively that of the contractor.  

29)
The Contractor shall obtain licence under CL(R&A) Act, 1970. 










ANNEXURE-D

GENERAL TERMS & CONDITIONS

1. BHEL shall have the privity of the contract with the contractor only only and will give instructions to the contractor or his authorized representative. BHEL will have nothing to do or be concerned with the employment of employees working for the contractor.  The relationship between BHEL and the Contractor will be that of independent entities and nothing herein contained will amount to joint venture, partnership or an employer-employee relationship.  

2. The contractor shall maintain regular contact with the designated employee(s) of BHEL and will interact on matters relating to the  work awarded under this contract.

3. In case the Contractor does not carry out the Contractual/Statutory obligations or the services rendered by him are found to be unsatisfactory, BHEL shall bring the same to his notice and he will be obliged to discharge the obligations and rectify the deficiency/anomaly within three days time failing which, BHEL reserves the right to terminate the contract without assigning any reason whatsoever.  In such an event, no damages will be payable for short closure of the contract.

4. The contractor shall deposit an amount of Rs.__________      ( 10% of contract value)  as security with BHEL in the form of pay order /bank guarantee / FDI in the name of contractor A/c – BHEL duly discharged on the back.  Alternatively, the amount of security deposit can be adjusted partly from EMD and balance amount from subsequent bills.  This security deposit shall be liable to be returned to the contractor after termination of the contract or at the end of it, subject to deduction on account of company dues, non-deposit of statutory dues etc.  No interest shall be payable on the  security deposit.

5. The decision of BHEL regarding interpretation of any of terms and conditions set forth in this agreement shall be final and binding on the Contractor.

6. Any matter arising out of or in connection with the agreement shall be under jurisdiction of SANGAREDDY/ HYDERABAD Court.

7. Notwithstanding anything contained in this Agreement, the contract may be terminated by BHEL without assigning any reason thereof by giving a notice of 30 days to the contractor. 

8. The contract will commence on ____________ date and will remain valid for a period of  One year till __________ date.  The parties reserve the right to extend the contract on mutually agreed terms and conditions.

9.
All disputes between the parties to the contract arising out of or relating to the contract either concerning the terms and conditions of the contract or its implementation will be settled by the parties amicably.  In case the dispute is not resolved, the same shall be referred for conciliation by an officer of BHEL.  In case it still remains unresolved, the dispute will be referred for arbitration by an officer of BHEL.  (In case arbitration by an officer of BHEL is not acceptable, the same could be entrusted to any mutually agreeable Government functionary or an independent person). The venue of arbitration shall be office of General Manager/ M&S (indicate place).  The arbitrator will give his award within a period of four
TERMS AND CONDITIONS ( should read with Schedule of work):

General:
1) Contractor should maintain logbook for the work attended on today basis.  Areas attended should be shown to Sr.Engineer/ M&S(Instrumentation), concerned area incharge & Inspector( Fire Services- CISF)  or their authorized persons  and should obtain signatures in the log book immediately 

2) Man power engaged should have required basic skills to attend the fire alarm systems at ground level or  placed at  heights.

3) Contractor should be given personnel protective equipments to workmen  and should see that they wore at work. 

4) penalty  will be levied at 1/2% per week on the total contract value to the maximum of 10% if AMC  is not attended at regular intervals once after issue of LOI
5) The contractor shall pay minimum wages to the staff as per minimum wages act applicable from time to time.    Contractor has to make PF, ESI contributions as per applicable rules; contractor should pay double wages if engaged on overtime beyond normal working hours.

6) Contractor should supervise the labour engaged for this work.
7) Contractor shall arrange ESI cards for the contract labour   and shall extend all relevant statutory rules for his staff.

8) Contractor shall comply the provision of PF, ESI for their labour. 

9) Contractor & their labour should attend safety classes being conducted by safety department.

10)   Contract operates on Unit rates, no revision of rates will be allowed during contract period.
11) The contractor is wholly responsible for any loss of life or partial disability of any of their employees while on duty

12) In case of occurrence of any accident/ injury of contractor’s staff, BHEL will not pay any compensation while they are on duty and contractor has to take care of same under statutory obligation.

13) Upon awardable of the work, the party has to execute an agreement with BHEL as per General Agreement Conditions before commencement of work, contractor should approach with prescribed stamp paper.

14) Read the specifications/ work content carefully before quote. 

15)  Contractor shall follow General  instructions/ Obligations of the contractors  
16) BHEL reserve the right to reject any or all tenders in part or in full without assigning any reason
17) Special conditions (technical) are enclosed separately.

18) Unit  rate should be inclusive of all taxes and duties should be indicated clearly in the unit rate column of Annexure-I,  rate of tax if applicable should be indicated separately

19) Contractor should make their own arrangements to report in BHEL for periodical checking  
20) Progressive payments will be arranged.  Bill should indicate the item name, quantity billed during that period.    Bill can be put up for once in a quarter year  period.  Payment will be released for  satisfactory  completion of work  in all respects  certification from user dept is required.  Certified copy  obtained from concerned departments should  be handed over to fire services department immediately by the contractor.
21) Bill should  be forwarded to   Sr.ENGINEER/ M&S(INST- 90 BLDG) with certified copies & Request for movement pass shall be addressed to Sr.ENGINEER/ M&S(INST- 90 BLDG)
22) BHEL working Hours:   1st shift: 06.45 to 15.15Hrs , 2nd shift: 15.00 to 23.30Hrs,  
           EMD can  be paid in cash at BHEL Cash office;   or by Demand Draft/ Bankers cheque drawn in the name of M/s Bharat Heavy Electricals Limited” payable at Hyderabad and shall be enclosed to the tender bid. No other means of payment will be accepted, no interest on EMD will be payable.         Cash office timings:   09.30 AM  to 11.30 AM  &   1.30 PM to 3.30 PM

PRICE BID

Anexure-2/1

	SCHEDULE OF WORK    FOR "ANNUAL MAINTENANCE 
	
	
	

	                  SERVICE CONTRACT  FOR FIRE ALARM   AND HALON EXTINGUISHING SYSTEM 

	                                       AT COMPUTER BUILDING "
	
	
	
	

	S.NO
	DESCRIPTION OF WORK
	QTY 
	UNIT
	RATE 
	AMOUNT

	 
	 
	 
	 
	Rs
	Rs


	1
	Maintenance of Fire Detection  and Alarm System
	
	
	
	

	
	in total consisting of the following equipments .
	 
	 
	
	

	A
	Checking  and testing of  the 24 Zone main  fire alarm 
	1
	set
	
	

	
	control panel with 8 zone Halon fire unit thoroughly by 
	 
	 
	
	

	
	simulation of all the zones from field,cheking and testing of 
	 
	 
	
	

	
	batterypack for backup healthyness,  replacement of any 
	 
	 
	
	

	
	components with genuine spares.The contractor shall 
	 
	 
	
	

	
	replace the defective spares with genuine spares of  OEM
	 
	 
	
	

	
	of make WELS Make.
	 
	 
	
	

	B
	Checking and testing of 14 Zone Fire Alarm control
	1
	Set
	
	

	
	panel thoroughly  by simulation  of all zones from field,
	 
	 
	
	

	
	checking and testing of battery pack for backup,
	 
	 
	
	

	
	replacement of  defective components/ spares with 
	 
	 
	
	

	
	genuine components of ADEMCO make.
	 
	 
	
	

	C
	Checking & testing of smoke ionization and smoke optical  
	58
	Nos
	
	

	
	detectors  of make Apollo,UK  thoroughly by simulation.
	 
	 
	
	

	D
	Checking & testing of system sensors and detectors of 
	25
	Nos
	
	

	
	make USA  thoroughly by simulation .  
	 
	 
	
	

	E
	Checking of manual call points and simulation of the 
	3
	Nos
	
	

	
	conditions thoroughly and replacement of  manual
	 
	 
	
	

	
	call points which are defective. 
	 
	 
	
	

	F
	Checking and testing of manual call break  glass  and 
	10
	Nos
	
	

	
	push buttons  for halon, simulation of the conditions 
	 
	 
	
	

	
	and replacement of defective  (pill box) for Halon.
	 
	 
	
	

	G
	Simulation of the conditions and testing  of  electronic 
	4
	Nos
	
	

	
	hooter for minimum two times for a span of five minutes
	 
	 
	
	

	
	and repacement of the item if found defective.
	 
	 
	
	

	H
	Checking the circuit of Halon extinguisher system 
	3
	Nos
	
	

	
	of the cylinder  of capacity  50 Kg. refilling if required.
	 
	 
	
	

	I
	Checking the circuit of Halon extinguisher system and 
	5
	Nos
	
	

	
	refilling of the cylinder  of capacity 25 Kg.
	 
	
	
	

	
	 
	 
	
	
	

	J
	Checking and testing of DL Trip box  
	1
	No
	
	

	K
	Checking and testing of   Halon control  panel by 
	1
	Set
	
	

	
	simulation.
	 
	 
	
	

	L
	Checking and testing of smoke detectors  
	15
	Nos
	
	

	
	by simulation  and replacement, if found defective.
	 
	 
	
	










RUPEES
_________

Total amount in rupees ( words)

Anexure-2/2

	            SCHEDULE OF WORK   FOR "ANNUAL MAINTENANCE SERVICE CONTRACT  
	

	
	FOR FIRE ALARM  AND HALON EXTINGUISHING SYSTEM AT TELEPHONE EXCHANGE "

	
	
	
	
	
	

	S.NO
	DESCRIPTION OF WORK
	QTY 
	UNIT
	RATE 
	AMOUNT

	 
	 
	 
	 
	Rs
	Rs

	I
	Maintenance of Fire Detection  and Alarm System 
	 
	 
	 
	 

	 
	in total consisting of the following equipments.
	 
	 
	 
	 

	 
	 
	 
	 
	
	 

	(a)
	Checking  and testing of  the 24 Zone main  fire alarm 
	1
	set
	
	

	 
	control panel with 4 zone Halon fire unit thoroughly by 
	 
	 
	
	

	 
	simulation of all the zones from field, cheking and testing
	 
	 
	
	

	 
	of batterypack for backuphealthyness,  replacement of 
	 
	 
	
	

	 
	any components with genuine spares by the contractor 
	 
	 
	
	

	 
	 OEM (ADEMCO, UK ).
	 
	 
	
	

	(b)
	Checking of  smoke  ionization  &  smoke optical
	57
	Nos
	
	

	 
	sensors/detectors  of make System sensors ,USA 
	 
	 
	
	

	 
	thoroughly by simulation.
	 
	 
	
	

	(c )
	Checking the circuit of  Halon extinguisher system
	4
	Nos
	
	

	 
	and refilling of Halon gas of capaity 25 Kgs.
	 
	 
	
	

	(d)
	Checking of manual call points and simulation of the 
	3
	Nos
	
	

	 
	conditions thoroughly and replacement of  manual
	 
	 
	
	

	 
	call points which are defective. 
	 
	 
	
	

	(e)
	Checking and testing of manual call break  glass  and 
	8
	Nos
	
	

	 
	push buttons  for halon, simulation of the conditions 
	 
	 
	
	

	 
	and replacement of defective  (pill box) for Halon.
	 
	 
	
	

	(f)
	Simulation of the conditions and testing  of  electronic 
	3
	Nos
	
	

	 
	hooter for minimum two times for a span of five minutes
	 
	 
	
	

	 
	and repacement of the item if found defective.
	 
	 
	
	

	(g)


	Checking and testing of Halon control  panel by simulation.


	1


	No.


	
	

	 (h)
	Checking and testing by simulation of all the smoke
	10 
	Nos 
	 
	

	 
	detectors and replacement if found defective.
	 
	 
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	 
	
	TOTAL AMOUNT  
	


Total amount in rupees ( words)

Anexure-2/3

	  SCHEDULE OF WORK  FOR "ANNUAL MAINTENANCE 
	
	
	

	SERVICE CONTRACT  FOR FIRE ALARM  AND DETECTION SYSTEM AT 06 ANNEXE 

	                                ARCHIVE AND REPROGRAPHICS SECTION"
	
	

	
	
	
	
	
	

	S.NO
	DESCRIPTION OF WORK
	QTY 
	UNIT
	RATE 
	AMOUNT

	 
	 
	 
	 
	Rs
	Rs

	I
	Maintenance of Fire Detection  and Alarm System 
	 
	 
	 
	 

	 
	in total consisting of the following equipments.
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	(a)
	Checking  and testing of  the 20 Zone main  fire alarm 
	1
	set
	
	

	 
	control panel thoroughly by simulation of all the zones
	 
	 
	
	

	 
	from field, cheking and testing of batterypack for backup
	 
	 
	
	

	 
	healthyness,  replacement of any components with
	 
	 
	
	

	 
	 genuine spares. The contractor shall replace
	 
	 
	
	

	 
	 the defective spares with genuine spares of  OEM.
	 
	 
	
	

	 
	 
	 
	 
	
	

	(b)
	Checking of  smoke  ionization  &  smoke optical
	40
	Nos
	
	

	 
	sensors/detectors  of make System sensors, USA 
	 
	 
	
	

	 
	thoroughly by simulation. 
	 
	 
	
	

	 
	 
	 
	 
	
	

	(c )
	Checking of manual call points and simulation of the 
	2
	Nos
	
	

	 
	conditions thoroughly and replacement of  manual
	 
	 
	
	

	 
	call points which are defective. 
	 
	 
	
	

	 
	 
	 
	 
	
	

	(d)
	Simulation of the conditions and testing  of  electronic 
	2
	Nos
	
	

	 
	hooter for minimum two times for a span of five minutes
	 
	 
	
	

	 
	and replacement of the item if found defective.
	 
	 
	
	

	 
	 
	 
	 
	
	

	(e)
	Checking, testing  by simulation of all the smoke 
	10
	Nos
	
	

	 
	detectors and replacement if found defective 
	 
	 
	
	

	 
	 
	 
	 
	
	

	 
	TOTAL  AMOUNT  Rs.

Rupees in words: 


	 
	 
	
	


Anexure-2/4
	SCHEDULE OF WORK   FOR "ANNUAL MAINTENANCE SERVICE CONTRACT 
	
	

	 FOR
	FIRE ALARM  AND HALON EXTINGUISHING SYSTEM AT  TC&GT TEST PIT AREA INCLUDING TESTING FRAME-9, TESTING FRAME-6, TESTING FRAME-3, TESTING FRAME-5 "

	
	
	
	
	
	

	S.NO
	DESCRIPTION OF WORK
	QTY 
	UNIT
	RATE 
	AMOUNT

	 
	 
	 
	 
	Rs
	Rs

	I
	TESTING FRAME 9 AREA :
	 
	 
	 
	 

	 
	Maintenance of Fire Detection  and Alarm System 
	 
	 
	 
	 

	 
	in total consisting of the following equipments .
	 
	 
	 
	 

	 
	 
	 
	 
	 
	 

	(a)


	Checking  and testing of  the 12 Zone main  fire alarm control panel  with 6 Zone Halon fire unit
	1
	set
	
	

	 
	thoroughly by simulation of all the zones from field,
	 
	 
	
	

	 
	checking and testing of battery pack for backup
	 
	 
	
	

	 
	Healthy ness,  replacement of any components with genuine spares.The contractor shall replace the
	 
	 
	
	

	 
	 defective spares with genuine spares of OEM  make
	 
	 
	
	

	(b)
	Checking of  smoke  ionization  sensors/detectors
	23
	Nos
	
	

	 
	 of make Apollo ,UK  thoroughly by simulation. 
	 
	 
	
	

	(c)
	Checking of  smoke  optical sensors and detectors
	6
	Nos
	
	

	 
	of make System Sensors, USA thoroughly by simulation.
	 
	 
	
	

	(d)


	Checking of  Heat Detectors thoroughly by simulation and replacement of  defective detectors
	2
	Nos
	
	

	(e)


	Checking of manual call points  (pill box) and simulation of the  conditions thoroughly and replacement of  manual call points which are defective
	3


	Nos


	
	

	(f)


	Checking and testing of manual call break  glass  and push buttons (pill box) for halon,simulation
	10


	Nos


	
	

	 
	of the conditions and replacement of defective  
	 
	 
	
	

	 
	 (pill box) for Halon.
	 
	 
	
	

	(g)


	simulation of the conditions and testing  of  electronic  hooter for minimum two times for a
	4
	Nos
	
	

	 
	span of five minutes and replacement of the
	 
	 
	
	

	 
	item if found defective.
	 
	 
	
	

	(h)


	Checking the circuit of Halon extinguisher system and refilling of the cylinder  of capacity 25 Kg.
	3
	Nos
	
	

	(I)
	Checking  and testing of Halon  release buttons for 
	3
	Nos
	
	

	 
	efficient operation.
	 
	 
	
	

	(j)
	Checking and testing of Halon control  panel by 
	1
	Set
	
	

	 
	simulation.
	 
	 
	
	

	 
	 
	 
	 
	
	

	(k)
	Checking and testing of smoke detectors  (optical )
	7
	Nos
	
	

	 
	by simulation  and replacement  if found defective.
	 
	 
	
	

	 
	
	
	AMOUNT 
	Rupees
	

	 
	
	
	

	 
	 AMOUNT IN RUPEES( WORDS)
	 
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	ANNEXURE- 2/5
	
	

	
	FIRE ALARM AMC FOR TEST PIT AREA CONT…
	
	
	
	

	S.NO
	DESCRIPTION OF WORK
	QTY 
	UNIT
	RATE 
	AMOUNT

	 
	 
	 
	 
	Rs
	Rs

	I
	TESTING FRAME-6  AREA :
	 
	
	
	

	 
	Maintenance of Fire Detection  and Alarm System 
	 
	
	
	

	 
	in total consisting of the following equipments .
	 
	
	
	

	 
	 
	 
	
	
	

	(a)


	Checking  and testing of  the 12  Zone main  fire alarm control panel with 4 zone Halon fire unit
	1
	SET
	
	

	 
	  thoroughly by simulation of all the zones from
	 
	
	
	

	 
	field, cheking and testing of batterypack for backup
	 
	
	
	

	 
	healthyness,  replacement of any components
	 
	
	
	

	 
	with genuine spares.The contractor shall replace  the defective spares with genuine spares of  
	 
	
	
	

	 
	OEM make.
	 
	
	
	

	 
	 
	 
	
	
	

	(b)
	Checking of  smoke  ionization  sensors/detectors
	23
	Nos
	
	

	 
	 of make Apollo, UK  thoroughly by simulation. 
	 
	
	
	

	 
	 
	 
	
	
	

	(c)
	Checking of  smoke  optical sensors  
	6
	Nos
	
	

	 
	and detectors  of make System Sensors, USA  
	 
	
	
	

	 
	thoroughly by simulation.  
	 
	
	
	

	 
	 
	 
	
	
	

	(d)


	Checking of manual all call points and simulation of the conditions thoroughly and replacement
	3
	Nos


	
	

	
	 
	 
	
	
	

	(e)


	Checking and testing of manual call break  glass  and push buttons  for halon , simulation of
	10
	Nos
	
	

	 
	the conditions and replacement of defective  (pill box) for Halon.
	 
	
	
	

	 
	 
	 
	
	
	

	(f)


	simulation of the conditions and testing  of  electronic hooter for minimum two times for hooter for minimum two times for a span of five minutes
	4
	Nos
	
	

	 
	and replacement of the item if found defective.
	 
	
	
	

	 
	 
	 
	
	
	

	(g)


	Checking the circuit of Halon extinguisher system and refilling of the cylinder  of capacity 54 Kg.
	2
	Nos
	
	

	(h)
	Checking  and testing of Halon  release buttons for 
	2
	Nos
	
	

	 
	efficient operation.
	 
	
	
	

	(i)
	Checking and testing of Halon control  panel by 
	1
	Set
	
	

	 
	simulation.
	 
	
	
	

	 
	 
	 
	
	
	

	(j)
	Checking and testing of smoke detectors  (optical )
	7
	Nos 
	
	

	 
	by simulation  and replacement  if found defective.
	 
	
	
	

	 
	
	
	
	
	

	
	
	AMOUNT RS.
	

	
	AMOUNT IN RUPEES( WORDS)

ANNEXURE- 2/6
	
	
	
	

	
	FIRE ALARM AMC FOR TEST PIT AREA CONT…
	
	
	
	

	S.NO
	DESCRIPTION OF WORK
	QTY 
	UNIT
	RATE 
	AMOUNT

	 
	 
	 
	 
	Rs
	Rs

	III
	TESTING FRAME 3  &  5 AREA :
	 
	
	
	

	 
	Maintenance of Fire Detection  and Alarm System 
	 
	
	
	

	 
	in total consisting of the following equipments .
	 
	
	
	

	 
	 
	 
	
	
	

	(a)


	Checking  and testing of  the 14  Zone main  fire alarm control panel with 5 zone Halon
	1
	SET
	
	

	 
	fire unit thoroughly by simulation of all the zones
	 
	
	
	

	 
	from field, cheking and testing of batterypack for backup healthyness,  replacement of any
	 
	
	
	

	 
	spares.The contractor shall replace  the defective
	 
	
	
	

	 
	spares with genuine spares of  OEM make.
	 
	
	
	

	(b)
	Checking of  smoke  ionization  sensors/detectors
	23
	NOS
	
	

	 
	 of make Apollo ,UK  thoroughly by simulation. 
	 
	
	
	

	(c)


	Checking of  smoke  optical sensors and detectors of make  System Sensors, USA
	6
	NOS
	
	

	 
	thoroughly by simulation.  
	 
	
	
	

	(d)


	Checking and testing of heat detectors thoroughly by simulation and  replacement of
	2
	NOS
	
	

	 
	the items if defective.
	 
	
	
	

	(e)
	Checking of manual call points and simulation of the 
	1
	NO
	
	

	 
	conditions thoroughly and replacement of  manual
	 
	
	
	

	 
	call points which are defective. 
	 
	
	
	

	(f)


	Simulation of the conditions and testing  of  electronic hooter for minimum two times for a span
	1
	NO
	
	

	 
	of five minutes hooter for minimum two times and replacement of the item if found defective.
	 
	
	
	

	(g)


	Checking the circuit of Halon extinguisher system and refilling of the cylinder  of capacity 25 Kg
	4
	NOS
	
	

	(h)


	Checking the circuit of Halon extinguisher system of the cylinder  of capacity 50 Kg
	1
	NO
	
	

	(I)
	Checking  and testing of response  indicators.
	8
	NOS
	
	

	(j)
	Checking and testing of Halon control  panel by 
	1
	SET
	
	

	 
	simulation.
	 
	
	
	

	(k)
	Checking and testing of smoke detectors  (optical )
	10
	NOS
	
	

	 
	by simulation  and replacement  if found defective.
	
	
	
	

	(L)


	Checking & testing of Halon release inhibit and simulation of the conditions thoroughly 
	5
	NOS
	
	

	 
	and replace if found  defective. 
	
	
	
	

	 
	 
	TOTAL  
	RUPEES
	
	

	 
	AMOUNT IN RUPEES( WORDS)
	 
	
	
	


ANNEXURE- 2/7
	
	AMC FOR FIRE ALARM AND CONTROL SYSTEM AT MRS AND 10 PANEL  ROOM
	
	
	
	

	S.NO
	DESCRIPTION OF WORK
	QTY 
	UNIT
	RATE 
	AMOUNT

	 
	 
	 
	 
	Rs
	Rs

	I
	
	 
	
	
	

	 
	Maintenance of Fire Detection  and Alarm System 
	 
	
	
	

	 
	in total consisting of the following equipments .
	 
	
	
	

	 
	 
	 
	
	
	

	(a)


	Checking  and testing of  NOTIFIER make  main  fire alarm control panel unit model NSF-640 thoroughly
	1
	SET
	
	

	 
	by simulation of all the  7 zones from field, checking
	 
	
	
	

	 
	and testing of battery pack for backup healthyness,  replacement of any components with genuine spares.
	 
	
	
	

	 
	of  NOTIFIER make.
	 
	
	
	

	(b)
	Checking and testing of  fault  isolator thoroughly  
	1
	NO
	
	

	 
	 by simulation. 
	 
	
	
	

	 
	 
	 
	
	
	

	(c)


	Checking of  photo electric type  smoke sensors  of make  Notifier, thoroughly by simulation.  
	24
	NOS
	
	

	 
	 
	 
	
	
	

	(d)


	Checking and testing of auto  dialor ( Model TD-110)  thoroughly by simulation 
	2
	NOS
	
	

	(e)
	Checking of manual call points and simulation of the 
	3
	NO
	
	

	 
	conditions thoroughly and replacement of  manual
	 
	
	
	

	 
	call points which are defective. 
	 
	
	
	

	(f)


	Simulation of the conditions and testing  of  electronic hooter for minimum two times for a span of five minutes
	2
	NO
	
	

	 
	and replacement of the item if found defective.
	 
	
	
	

	 
	 
	 
	
	
	

	(g)


	Checking and testing of Mimic  panel for zone identification
	1
	SET
	
	

	(h)
	Checking and testing of smoke detectors  (optical )
	2
	NOS
	
	

	 
	by simulation  and replacement  if found defective.
	
	
	
	


TOTAL AMOUNT RUPEES

AMOUNT IN RUPEES( WORDS)
TOTAL FOR ANNEXURES 1 to  7 :  Rs.

( in words Rupees

specify the amount arrived is  with or without taxes, duties etc.
