	PSGSG/12-13/022
	Product Specifications

For

STC TESTING
	Drg. No.
	-

	
	
	Date
	12.11.14

	
	
	Product
	GSM 245

	     1.0

2.0

3.0

4.0

5.0
	Application: STC testing of 245kV current transformer for 40kA/50kA, for 3 Sec.
 Test Object:  245kV current transformer.

 Quantity       : 1. 245kV, 40kA CT for 3 sec        1 no.

                       2. 245kV, 50KA CT for 3 sec        1 no.
Following test to be carried out on Current Transformer Module:
Test description
No. of samples

STC 40kA for 3 Sec.

1 nos.
STC 50kA for 3 Sec.
1 nos.

Accuracy check for 0.2 class at 100% & 25% burden in pre-post test 

02 nos.
Accuracy check for PS/5P20 class 

02 nos.
Instrumental security factor in pre-post tests

04 nos.
Over-voltage inter turn test

04 nos.
Resistance measurement of windings

04nos.
Measurement of Knee point voltage

02 nos.
GENERAL :



In case of doubts in specifications the supplier shall contact BHEL for clarifications.



	1/1
	PSGSG/12-13/022.doc


	
	Signature


